SIMULATION EXPERIMENT – 6

Simulation-Based Investigation of Microwave Semiconductor Devices and
Frequency-Conversion Characteristics of a Microwave Mixer

Course: ECA1304 – Microwave Engineering

Name : Mohit V

Reg No : 192512043

# Aim

To simulate the operating characteristics of microwave semiconductor devices, analyze their electrical and frequency-dependent behavior, investigate frequency conversion in a microwave mixer, and evaluate conversion performance under specified RF and LO power conditions.

# Software Used

MATLAB (for numerical modeling and plotting). The programs use illustrative analytical models for semiconductor and mixer behavior. ADS, LTspice, or Multisim may be used for circuit-level simulation; full device-accurate results require suitable device models and circuit parameters.

# Design Specifications

| Parameter | Setting | Target / Expected |
| --- | --- | --- |
| RF frequency | 10 GHz | 10 GHz |
| LO frequency | 9.5 GHz | 9.5 GHz |
| RF input power | −10 dBm | −10 dBm |
| LO power | +7 dBm | +7 dBm |
| IF load | 50 Ω | 50 Ω |
| Difference frequency | ¦fRF − fLO¦ | 0.5 GHz |
| Sum frequency | fRF + fLO | 19.5 GHz |
| Passive mixer conversion loss | Model dependent | Approximately 6–10 dB |

# Objective 1: Schottky Diode I–V Characteristics

Aim: To develop a simulation model representing the nonlinear current-voltage characteristics of a microwave Schottky diode.

## MATLAB Program

% Objective 1: Schottky Diode I-V Characteristics

clc; clear; close all;

Is = 1e-12;

n = 1.05;

Vt = 0.026;

V = -0.2:0.001:0.5;

I = Is*(exp(V/(n*Vt))-1);

plot(V,I,'LineWidth',2);

grid on;

xlabel('Voltage (V)');

ylabel('Current (A)');

title('Schottky Diode I-V Characteristics');

## Output / Observation

The current is very small at low and negative voltages.

Forward current increases rapidly as the applied voltage rises.

The plotted curve represents the nonlinear diode I–V characteristic.

Result: The Schottky diode I–V characteristic was modeled and plotted. The curve shows the expected exponential rise in forward current for the selected illustrative parameters.

Figure 1: MATLAB plot of Schottky diode I–V characteristics.

# Objective 2: Frequency-Dependent Semiconductor Characteristics

Aim: To analyze the frequency-dependent response of a microwave semiconductor device using a simplified series-resistance and junction-capacitance model.

## MATLAB Program

% Objective 2: Schottky Diode Frequency Response

clc; clear; close all;

Rs = 5;

Cj = 0.2e-12;

f = logspace(6,11,1000);

H = 1 ./ sqrt(1 + (2*pi*f*Rs*Cj).^2);

semilogx(f/1e9,20*log10(H),'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Schottky Diode Frequency Response');

## Output / Observation

The response remains close to 0 dB at lower frequencies.

The response decreases as frequency increases.

The trend illustrates the influence of series resistance and junction capacitance.

Result: The modeled frequency response was plotted on a logarithmic frequency axis. The simplified model demonstrates a gradual high-frequency roll-off.

Figure 2: MATLAB plot of the modeled Schottky diode frequency response.

# Objective 3: Microwave Mixer Frequency Conversion

Aim: To simulate the mixing of RF and LO signals and identify the difference- and sum-frequency components.

## MATLAB Program

% Objective 3: Microwave Mixer Frequency Conversion

clc; clear; close all;

fRF = 10e9;

fLO = 9.5e9;

Fs = 100e9;

t = 0:1/Fs:100e-9-1/Fs;

RF = cos(2*pi*fRF*t);

LO = cos(2*pi*fLO*t);

IF = RF .* LO;

N = length(IF);

Y = abs(fft(IF))/N;

f = (0:N-1)*(Fs/N);

plot(f(1:floor(N/2))/1e9, ...

2*Y(1:floor(N/2)),'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Amplitude');

title('Microwave Mixer Output Spectrum');

xlim([0 25]);

fprintf('Difference frequency = %.2f GHz\\n',abs(fRF-fLO)/1e9);

fprintf('Sum frequency = %.2f GHz\\n',(fRF+fLO)/1e9);

## Output

Difference frequency = 0.50 GHz

Sum frequency = 19.50 GHz

Result: The product of the RF and LO signals produces the expected difference component at 0.5 GHz and sum component at 19.5 GHz. This is an idealized multiplication model; practical mixer output levels depend on the device and circuit.

Figure 3: MATLAB output spectrum showing the mixer conversion products.

# Objective 4: Conversion Loss and LO Power Dependence

Aim: To evaluate mixer conversion performance and examine the effect of LO power on the modeled IF output.

## MATLAB Program

% Objective 4: Conversion Loss and LO Power Dependence

clc; clear; close all;

PRF = -10;

PLO = -5:1:15;

% Illustrative conversion-loss model, minimum at +7 dBm LO

CL = 8 + 0.04*(PLO-7).^2;

PIF = PRF - CL;

plot(PLO,PIF,'o-','LineWidth',2);

grid on;

xlabel('LO Power (dBm)');

ylabel('IF Output Power (dBm)');

title('Mixer Conversion Performance');

k = find(PLO == 7);

fprintf('RF input power = %.2f dBm\\n',PRF);

fprintf('LO power = %.2f dBm\\n',PLO(k));

fprintf('Conversion loss = %.2f dB\\n',CL(k));

fprintf('IF output power = %.2f dBm\\n',PIF(k));

## Output

RF input power = -10.00 dBm

LO power = 7.00 dBm

Conversion loss = 8.00 dB

IF output power = -18.00 dBm

Result: In the illustrative model, the conversion loss at +7 dBm LO power is 8 dB, giving an IF output power of −18 dBm for a −10 dBm RF input. The curve is a chosen example, not a measured mixer response.

Figure 4: MATLAB plot illustrating IF output power versus LO power.

# Overall Result

The four MATLAB objectives were completed: the Schottky diode I–V characteristic was plotted, a simplified frequency response was analyzed, the mixer difference and sum frequencies were identified, and an illustrative conversion-loss model was evaluated. For the specified 10 GHz RF and 9.5 GHz LO signals, the theoretical IF is 0.5 GHz and the sum-frequency component is 19.5 GHz.

The semiconductor and mixer curves are illustrative mathematical models. They are useful for demonstrating trends and calculations but do not replace device-level circuit simulation. To verify actual conversion loss, spurious products, and power dependence, use a suitable mixer/diode model in ADS, LTspice, or Multisim and record the simulated IF output power.

# Formulae Used

Difference-frequency IF: fIF = |fRF − fLO|.

Sum-frequency component: fsum = fRF + fLO.

Conversion loss: CL = PRF,in − PIF,out, with both powers expressed in dBm.

IF output power: PIF,out = PRF,in − CL.

Percentage frequency deviation = |Practical − Theoretical| / |Theoretical| × 100%.

Idealized diode equation: I = Is[exp(V/(nVt)) − 1].