MOHIT V – 192512043 ECA1304 MICROWAVE ENGINEERING

EXP 1 Simulation-Based Electromagnetic Analysis of TEM, TE and TM Propagation Modes in Microwave Transmission Structures

AIM:

To simulate and analyze the TEM, TE₁₀ and TM₁₁ modes in microwave transmission structures using MATLAB, and to study their field distributions, cutoff frequencies and propagation characteristics.

PROCEDURE:

- Define the dimensions of the WR-90 rectangular waveguide:

  - a=22.86mm

  - b=10.16mm

  - Operating frequency f=10GHz.

- Calculate the cutoff frequency using:

fc=c2ma2+nb2

- Calculate the propagation constant:

β=2πfc1-fcf2

- Generate MATLAB plots to visualize the electric and magnetic field distributions for TE₁₀ and TM₁₁ modes.

- Compare the cutoff frequencies and propagation behavior of TE₁₀ and TM₁₁.

- Represent the TEM mode and compare its propagation characteristics with TE and TM modes.

- Analyze the obtained graphs and verify the results with theoretical values.

OBJECTIVES AND CODE

OBJECTIVE 1

clc;

clear;

close all;

c = 3e8;

f = 10e9;

% WR-90 dimensions

a = 22.86e-3;

b = 10.16e-3;

% TEM mode

beta_TEM = 2*pi*f/c;

% TE10 mode

fc_TE10 = c/(2*a);

beta_TE10 = 2*pi*f/c*sqrt(1-(fc_TE10/f)^2);

% TM11 mode

fc_TM11 = (c/2)*sqrt((1/a)^2+(1/b)^2);

if f > fc_TM11

beta_TM11 = 2*pi*f/c*sqrt(1-(fc_TM11/f)^2);

else

beta_TM11 = 1i*(2*pi*f/c)*sqrt((fc_TM11/f)^2-1);

end

fprintf('TEM beta = %.2f rad/m\n',beta_TEM);

fprintf('TE10 cutoff = %.3f GHz\n',fc_TE10/1e9);

fprintf('TE10 beta = %.2f rad/m\n',beta_TE10);

fprintf('TM11 cutoff = %.3f GHz\n',fc_TM11/1e9);

fprintf('TM11 beta = ');

disp(beta_TM11);

OBJECTIVE 2

clc;

clear;

close all;

a = 22.86e-3;

b = 10.16e-3;

x = linspace(0,a,100);

y = linspace(0,b,60);

[X,Y] = meshgrid(x,y);

% TE10 fields

E_TE10 = sin(pi*X/a);

H_TE10 = cos(pi*X/a);

% TM11 fields

E_TM11 = sin(pi*X/a).*sin(pi*Y/b);

H_TM11 = cos(pi*X/a).*cos(pi*Y/b);

% TE10 E-field

figure;

surf(X*1000,Y*1000,E_TE10);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('E-field');

title('TE10 E-field');

colorbar;

% TE10 H-field

figure;

surf(X*1000,Y*1000,H_TE10);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('H-field');

title('TE10 H-field');

colorbar;

% TM11 E-field

figure;

surf(X*1000,Y*1000,E_TM11);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('E-field');

title('TM11 E-field');

colorbar;

% TM11 H-field

figure;

surf(X*1000,Y*1000,H_TM11);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('H-field');

title('TM11 H-field');

colorbar;

OBJECTIVE 3

clc;

clear;

c = 3e8;

a = 22.86e-3;

b = 10.16e-3;

f = 10e9;

% TE10

m = 1;

n = 0;

fc_TE10 = (c/2)*sqrt((m/a)^2+(n/b)^2);

k0 = 2*pi*f/c;

beta_TE10 = k0*sqrt(1-(fc_TE10/f)^2);

% TM11

m = 1;

n = 1;

fc_TM11 = (c/2)*sqrt((m/a)^2+(n/b)^2);

if f > fc_TM11

beta_TM11 = k0*sqrt(1-(fc_TM11/f)^2);

else

beta_TM11 = 1i*k0*sqrt((fc_TM11/f)^2-1);

end

fprintf('--- RESULTS ---\n');

fprintf('TE10 cutoff frequency = %.3f GHz\n',fc_TE10/1e9);

fprintf('TE10 propagation constant = %.2f rad/m\n',beta_TE10);

fprintf('TM11 cutoff frequency = %.3f GHz\n',fc_TM11/1e9);

fprintf('TM11 propagation constant = ');

disp(beta_TM11);

OBJECTIVE 4

clc;

clear;

c = 3e8;

f = 10e9;

a = 22.86e-3;

b = 10.16e-3;

% TEM

fc_TEM = 0;

beta_TEM = 2*pi*f/c;

% TE10

fc_TE10 = c/(2*a);

beta_TE10 = 2*pi*f/c*sqrt(1-(fc_TE10/f)^2);

% TM11

fc_TM11 = (c/2)*sqrt((1/a)^2+(1/b)^2);

fprintf('MODE COMPARISON\n');

fprintf('-----------------------------\n');

fprintf('TEM  : fc = 0 GHz, beta = %.2f rad/m\n',beta_TEM);

fprintf('TE10 : fc = %.3f GHz, beta = %.2f rad/m\n',...

fc_TE10/1e9,beta_TE10);

fprintf('TM11 : fc = %.3f GHz\n',fc_TM11/1e9);

fprintf('\nPROPAGATION ANALYSIS\n');

if f > fc_TE10

fprintf('TE10 -> PROPAGATES\n');

else

fprintf('TE10 -> EVANESCENT\n');

end

if f > fc_TM11

fprintf('TM11 -> PROPAGATES\n');

else

fprintf('TM11 -> BELOW CUTOFF\n');

end

fprintf('\nPRACTICAL SUITABILITY\n');

fprintf('TEM  -> Coaxial transmission lines\n');

fprintf('TE10 -> Dominant mode in rectangular waveguides\n');

fprintf('TM11 -> Higher-order mode; not propagated at 10 GHz\n');

RESULTS

The TEM, TE₁₀ and TM₁₁ modes were successfully simulated using MATLAB.

- TE₁₀ cutoff frequency: ≈ 6.56 GHz

- TM₁₁ cutoff frequency: ≈ 16.42 GHz

- At 10 GHz, TE₁₀ propagates since f>fc.

- At 10 GHz, TM₁₁ is below cutoff and therefore does not propagate.

- TE₁₀ was identified as the dominant mode of the WR-90 waveguide.

- TEM mode has no cutoff frequency in an ideal TEM transmission line.

Thus, the field distributions and propagation characteristics of TEM, TE and TM modes were successfully analyzed.