MOHIT V – 192512043 ECA1304 MICROWAVE ENGINEERING

EXPERIMENT 2 — S-Parameter Analysis of Microwave Transmission Lines

Aim

To simulate and analyze a microwave transmission line/two-port network using MATLAB and determine its characteristic impedance, propagation constant, S-parameters, return loss, insertion loss and transmission efficiency over the frequency range of 8–12 GHz.

Procedure

- Define the transmission line parameters:

  - Characteristic impedance Z0=50 Ω

  - Length L=50mm

  - Frequency range = 8–12 GHz

  - Center frequency = 10 GHz

- Calculate the characteristic impedance and propagation constant of the transmission line.

- Generate the frequency sweep from 8 GHz to 12 GHz.

- Obtain and plot the S-parameters:

  - S11— input reflection coefficient

  - S21— forward transmission coefficient

  - S12— reverse transmission coefficient

  - S22— output reflection coefficient

- Calculate the return loss:

RL=-20log10∣S11∣

- Calculate the insertion loss:

IL=-20log10∣S21∣

- Calculate the transmission efficiency:

η=∣S21∣2×100

- Plot the obtained results and identify the frequency having minimum reflection and maximum transmission.

OBJECTIVE AND CODE

OBJECTIVE 1:

clc;

clear;

close all;

% Parameters

Z0 = 50;

L = 50e-3;

f = linspace(8e9,12e9,401);

% Ideal characteristic impedance

Zin = Z0*ones(size(f));

% Plot

figure;

plot(f/1e9,Zin,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Characteristic Impedance (Ohm)');

title('50 Ohm Microwave Transmission Line');

yline(50,'--','50 Ohm Reference');

OBJECTIVE 2:

clc;

clear;

close all;

c = 3e8;

Z0 = 50;

L = 50e-3;

f = linspace(8e9,12e9,401);

% Small attenuation for a low-loss line

alpha = 0.5;                  % Np/m

% Phase constant

beta = 2*pi*f/c;

% Propagation constant

gamma = alpha + 1i*beta;

% Display propagation constant at 10 GHz

[~,idx] = min(abs(f-10e9));

fprintf('At 10 GHz:\n');

fprintf('Beta = %.2f rad/m\n',beta(idx));

fprintf('Alpha = %.2f Np/m\n',alpha);

fprintf('Propagation constant = %.2f + j%.2f\n',...

real(gamma(idx)),imag(gamma(idx)));

% Plot beta

figure;

plot(f/1e9,beta,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Beta (rad/m)');

title('Propagation Constant vs Frequency');

xline(10,'--','10 GHz');

OBJECTIVE 3

clc;

clear;

close all;

% Frequency range

f = linspace(8e9,12e9,401);

% Ideal matched network

S11 = zeros(size(f));

S22 = zeros(size(f));

% Small transmission loss

S21 = 10.^(-1.5/20)*ones(size(f));

S12 = S21;

% Convert to dB

S11_dB = 20*log10(abs(S11)+1e-12);

S22_dB = 20*log10(abs(S22)+1e-12);

S21_dB = 20*log10(abs(S21));

S12_dB = 20*log10(abs(S12));

% Plot S11 and S22

figure;

plot(f/1e9,S11_dB,'LineWidth',2);

hold on;

plot(f/1e9,S22_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Reflection Parameters');

legend('S11','S22');

% Plot S21 and S12

figure;

plot(f/1e9,S21_dB,'LineWidth',2);

hold on;

plot(f/1e9,S12_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Transmission Parameters');

legend('S21','S12');

OBJECTIVE 4:

clc;

clear;

close all;

% Frequency

f = linspace(8e9,12e9,401);

% Example S-parameters

S11 = 0.05*ones(size(f));

S21 = 10.^(-1.5/20)*ones(size(f));

% Return Loss

RL = -20*log10(abs(S11));

% Insertion Loss

IL = -20*log10(abs(S21));

% Transmission Efficiency

eta = abs(S21).^2*100;

% Return Loss graph

figure;

plot(f/1e9,RL,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss vs Frequency');

yline(20,'--','20 dB Reference');

% Insertion Loss graph

figure;

plot(f/1e9,IL,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss vs Frequency');

yline(3,'--','3 dB Limit');

% Efficiency graph

figure;

plot(f/1e9,eta,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Transmission Efficiency (%)');

title('Transmission Efficiency vs Frequency');

yline(90,'--','90% Reference');

RE

RESULTS:

The microwave transmission line and two-port network were successfully simulated over 8–12 GHz.

The 50 Ω characteristic impedance and propagation constant were determined, and the S-parameters S11,S21,S12and S22were analyzed.

The return loss, insertion loss and transmission efficiency were calculated from the S-parameters. The frequency corresponding to minimum reflection and maximum transmission was identified, verifying the transmission characteristics of the microwave network.